Report on OCAM 1B DFS matters

INTRODUCTION

The commissioning plan (doc nr) for OCAM1 has been  fully executed.

In addition to the plan PR images on M17, Omega Cent. and the Hercules cluster have been taken.

As the work is fully on schedule,  during  the last nights already an OCAM2 OB could be exercised: pointing the standard field SA110 on each of the 32 chips  in the g band.

The establishment of the settings of the amplifiers and final configuration of hardware  was achieved only 2 nights before the end of the run. This was not scheduled, but during the last two days and nights most critical calibration observations could be repeated, leading to a first internally consistent set. Observations taken earlier then the last two nights should be avoided for characterization purposes, when possible.

The various pipelines were successfully applied to the data and the data analysis was executed as planned. The unplanned much shorter period between OCAM1 and OCAM2 implied a prime attention to those issues possibly impacting on the OCAM2 programme. Also,  a full report was not planned at this state and should await the final characterization.  The present writings in the following sections are very preliminary and are a draft or incomplete, as we have higher priorities at this moment. In fact, the full internal documentation is residing on the consortium Wiki pages. We have copied  some of this information here.

Many of the detector/amplifyer test were already done in OCAM1A (see report doc nr) and indicated an overall very good performance, conform the lab testing in Garching (see report doc nr).  No major non-conformances were noted in COM1B, though the slightly higher readnoise (*** e/sec, *** ADU/sec) implies that for u band exposures extra attention should be paid to optimizing integration times and number of dithers (perhaps N=4 is preferred over N=5).

Significant crosstalk was observed between CCDs *** (FIERRA) .  First analysis indicates that these can be suppressed quite will in the datahandling.

The overall ghosting is certainly not worse than expected. A few anomalies were observed and further characterized when a bright star is positioned on or near the crosses of segmented filters, or near, but outside, the edge of the field (the latter was experimentally confirmed to be due to *** at *** mm of the edge of the field).

In OCAM1B the instrument was for the very first time exposed to the dome screen of the calibration lamps (calibration unit) and the twilight sky. A thorough analysis indicates:

- The illumination of the screen by the lamps is quite homogenous,  better than 5% and with little straylight gradients

- The illumination in u is very poor, as expected,  (no output of lamp) and dominated by straylight-  its useage in u is questionable.

- The overall performance of the calibration unit is very good and promising

- Considerable straylight gradients on the sky have been deduced from the data: at least 15% (center-corner) circular symmetric  in g and with more substructure, non-symmetric but with a smaller amplitude in r,i,z . In B,V the straylight gradients are much less. The interference filter are much more sensitive to the gradients, due to the input angle criterion of FPs.  Perhaps we are the first to observe with such large interference filters.

- Three different methods give consistent straylight gradient results, confirming that the ratio of raw dome-to-sky images provide a good insight on the sky straylight gradients.

Zeropoints have been derived from the standard  fields and are TBD compared to ETC and our own throughput estimates.

The OB’s of the calibration templates have been all handled over to Paranal staff and parameter values have been fine tuned and executed. The current and subsequent characterization observations were and are to be done with this set of  OBs/templates.

At large Zenit distance fields  with overlapping images have been observed as a start to settling optimal filed center separation, how much overlap is required to obtain photometric cross-checks over fields and what is the influence of the large  straylight gradients in the corners.

It was noted that for longer visit of a field at large ZD (45 degrees) it was difficult and time consuming to maintain the AO of the telescope. While DIM seeing reported  0.7-0.8 arcsec a lot of effort was required to achieve 1.1 arcsec images, while originally the observations gave 1.4 arcsec. Obviously, faster and better acquisition would help to improve on this. During OCAM2 the improvements on the VST AO and/or OmegaCAM AO will be accessed again.

The relatively long acquisition times and the uncertainties on the improvements thereoff dominate the effective verification of ETCs and this should wait for OCAM2. The zeropoints will be affected by the straylight gradients, but the strategy to characterize 32 indepedent chips is in our favour (as the gross of the affect will be absorbed in individual chip zeropoints).

For OCAM2 the original Com plan is applicable and will be executed as such. Extra attention will be given to observations putting standard field (SA107, SA 104 and ***) on 32 chips to miximize our understanding of the substantial straylight, the handling over of the real-time health check (lamp and monitor on the sky), large ZD fields and survey overlaps.   We will have plenty opportunity to observe the straylight from the moon.